# Instructions for Intermatic Digital Electronic Timer (page 1 of 2)

Note:  To manually turn the lights On or Off, press the clear window

Helpful Hint: If the timer does not work or if the window displays “noOps” (no operation), check that the battery is good, and that the unit is not jammed too tightly into the switch box too tightly.

- Definition of Buttons

MODE

ON/OFF EVENT

RESET

(Diagram of buttons behind plastic cover)

DAY

H+

M+

MODE BUTTON:   Push and  release to change the mode of the timer.  The current mode can be read along the top of the display.

CLK mode: To set or change the current time and day.

PGM mode:	To set, review or change programmed ON/OFF times.

AUTO mode:	Automatic timer will control lights.  Manual override will work.

AUTO RAND mode:	Automatic timer will control lights  with random offsets of  up to 15 minutes.  Manual override will work.

MAN mode:	Only manual switching will work.  Any programmed times will stay in memory but will not be active.

ON/OFF EVENT BUTTON:	Function depends on mode.

CLK mode:	Button does nothing

PRG Mode:	Button sets what will happen at the programmed time.

All Other  Modes: Manually turns lights on or off.  This button is pushed when the clear cover is pressed.

- RESET BUTTON:  Recessed to prevent accidental use, use this to erase all programs.

DAY BUTTON:  Has 2 functions, depending on the mode:

CLK mode:	Push and release to cycle through the days of the week to set the current day.

PGM mode:	Push and release to cycle through all programming options.  (Individual days, all weekdays, all week-end days.  Note:  The day button must be set before doing the hours and minutes.  If reprogramming, be sure to set the day button even if only resetting the times.

H+ BUTTON:	Push and release to cycle forward through the hours of the day.  Be sure AM and PM setting is correct.  It does not go backwards.  Note:  The day button must be set before doing the hours and minutes.  If reprogramming, be sure to set the day button even if only resetting the times.

M+ BUTTON:	Push and release to cycle forward through the minutes.  It does not go backwards.

- Instructions for Intermatic Digital Electronic Timer  (page 2 of 2)

- Programming Instructions

MODE

ON/OFF EVENT

RESET

(Diagram of buttons behind plastic cover)

M+

H+

DAY

- How to set or change the clock:

- Remove the clear cover/switch.

- Push and release MODE button until CLK is displayed.

- Push and release DAY button until correct day of the week is displayed.

Note:  The day button must be set before doing the hours and minutes.  If reprogramming, be sure to set the day button even if only resetting the times.

- Push and release H+ until the correct hour and AM/PM are correct.

- Push and release M+ button until the minutes are correct.

- Push and release MODE button to choose AUTO, AUTO RAND or MAN.

- Replace clear cover/switch.

- Push and release clear cover/switch to switch lights ON or OFF if desired.

- How to set, change or review ON/OFF timers:

- Remove clear cover/switch.

- Push and release MODE button until PGM is displayed.

- Push and release DAY button until desired day or group of days is displayed (usually all 7 days).

- Push and release H+ button until the desired hour and AM/PM is displayed for the event (On or OFF) to happen.

- Push and release M+ until desired minutes are displayed.

- Push and release  ON/OFF/EVENT button to choose the next event to set or change.

- Push and release MODE button to choose AUTO, AUTO RANDom, or MANual operation.

- Replace clear cover/switch.

Return to Top / Email A-1 Electric / Return to Main  Tel: (1)(908)766-1010