X-10 Timer Programming Instructions:

Some notes about this wireless control system.

- The X-10 system (X-10, Leviton, Radio Shack, Smart House) sends coded signals throughout the house on the power wires.

- Each switch has a house code (A, B, C, etc) and a number code (1, 2, 3, etc.) to identify the switch.

- Transmitter switches have to be set to send the proper coded signal to the receiver switch.

- Transmitter switches may be in the wall like normal switches, or plug-in controller or timer.

- The receiver switches are different from normal switches.  You have to push the switch to make it turn  on or off.

- There is a safety slider at the bottom of the switch that shuts the switch off completely, even if you try pushing the switch.  Normally, this slider should be to the right.

- If there is noise on the power lines from plug-in intercom systems, motors, etc., the signals may not reach the receiver switches.

To Set Clock: (just like setting a clock radio)

- Move slider switch in upper left to “Set Clock”, then use “Time” up and down buttons as necessary.

- When finished, return the slider switch to “NORMAL USE”

To Program Timer Settings:

- Move slider switch to program.

- Adjust clock to desired “ON” time, then press “1”- ON, “2”- ON etc.

(Note: For each code (1, 2, etc), you can have two “On” and two “Off” settings.  Even if you need only one “On” and one “Off” time, set the 2nd setting for 5 or 10 minutes after the desired time.  This way, if the first On or Off signal doesn’t reach the switch, the 2nd signal can follow up on it.)

- Adjust clock to desired “OFF: time, then press “1”- OFF, “2” – OFF, etc.

- When finished, return slider switch to “NORMAL USE”.

To Change or Reset timer on/off settings:

You must first Clear the old settings -

- Move slider switch in upper left, to “REVIEW”

- Press “1”- “ON” button, then press “CLEAR” twice to clear any “ON” settings:

- Repeat for “1” – “OFF” etc.

- Repeat for “2” – “ON” as necessary,

- etc.

# As the times are cleared, the clock will show 00:00 for the setting.

# Then return the slider switch to “NORMAL USE”

To Review the timer settings:

# Move slider switch to “REVIEW”

- Press the “1”-On and “1”- Off buttons to see what their settings are.

- Press the “2” – On and “2” – Off buttons to see what their settings are.

- When finished, return the slider switch to “NORMAL USE”.