Instructions for Intermatic Digital Electronic Timer Sundowner Model (page 1 of 3)

Note:  To manually turn the lights On or Off, press the clear window

Definition of Buttons  (Below is a diagram of the buttons behind plastic cover)

MODE

NEXT

ON/OFF EVENT

RESET

DAY/DST

H+

YEAR –

M+

ZONE+

YEAR+

(

MODE BUTTON:   Push and  release to change the mode of the timer.  The current mode can be read along the top of the display.

- CLK mode: To set or change the current time and day.

- PGM mode:	To set, review or change programmed ON/OFF times.

- AUTO mode:	Automatic timer will control lights.  Manual override will work until next program time setting.

- AUTO RAND mode:	Automatic timer will control lights  with random offsets of  up to 15 minutes.  Manual override will work until next program timer setting.

- MAN mode:	Only manual switching will work.  Any programmed times will stay in memory but will not be active.

NEXT/ON/OFF EVENT BUTTON:	Function depends on mode.

- CLK mode:	Button does nothing

- PRG Mode:	Button sets what will happen at the programmed time.

- All Other  Modes: Manually turns lights on or off.  (Pressing the clear window actually presses this button.)

- RESET BUTTON:  Recessed to prevent accidental use, use this to erase all programs.  Press while holding down NEXT/ON/OFF button to clear all previous programming.

DAY/DST BUTTON:  Has 3 functions, depending on the mode:

CLK mode:	Push and release to cycle through the days of the week to set the current day.

PGM mode:	Push and release to cycle through all programming options.  (Individual days, all weekdays, all week-end days.  Note:  The day button must be set before doing the hours and minutes.  If reprogramming, be sure to set the day button even if only resetting the times.

DST:  		Press to select automatic or manual Daylight Savings Time.

H+ BUTTON:	Push and release to cycle forward through the hours of the day.  Be sure AM and PM setting is correct.  It does not go backwards.  Note:  The day button must be set before doing the hours and minutes.  If reprogramming, be sure to set the day button even if only resetting the times.

M+ BUTTON:	Push and release to cycle forward through the minutes.  It does not go backwards.

- Programming Instructions  for Intermatic Digital Electronic Timer Sundowner Model

MODE

NEXT

ON/OFF EVENT

RESET

DAY/DST

H+

YEAR –

M+

ZONE+

YEAR+

(diagram of buttons behind plastic cover)

- How to set or change the clock:

- Remove the clear cover/switch.

- Push and release MODE button until CAL (calendar) is displayed.

- Push and release the ‘YEAR +’  or ‘YEARr –‘ button to display the correct year.

- Press ‘NEXT’ button.  Display will flash ‘DATE’

- Use the M+ and DAY buttons to select the current date.

Note:  The day button must be set before doing the hours and minutes.  If reprogramming, be sure to set the day button even if only resetting the times.

- Press ‘NEXT’ button. Display will alternate between ‘DST’ and ‘AUTO’ or between ‘DST’ and ‘MAN’.

- Press DST button to alternate between automatic Daylight Savings Time and Manual.

DST will automatically change the clock twice a year.  During Auto is selected, a small ‘d’ will show in the clock during the other modes.

- Press ‘NEXT’.  Display will alternate between ‘ZONE’ and ‘CENTR’  Use the ‘ZONE +’ button to select ‘North’, CENTR’ or ‘SOUTH.

- Press ‘NEXT’.  ‘SUNUP’ and an AM time will show.  If desired you can change the calculated Sunup time with the ‘H +’ and ‘M +’ buttons.

- Press ‘NEXT’.  ‘SNSET’ will and a PM time will show.

- Press ‘NEXT’ to accept the settings.

- Replace clear plastic cover.

How to set or change the clock:

- Remove clear plastic cover

- Press the MODE button until ‘CLK’ is displayed.

- Push and release H+ until the correct hour and AM/PM are correct.

- Push and release M+ button until the minutes are correct.

- Push and release MODE button to choose AUTO, AUTO RAND or MAN.

- Replace clear cover.

- Push and release clear cover/switch to switch lights ON or OFF if desired.

- How to set, change or review ON/OFF timers:

- Remove clear cover/switch.

- Push and release MODE button until PGM is displayed.

- Push and release DAY button until desired day or group of days is displayed (usually all 7 days).

- Push and release H+ button until the desired hour and AM/PM is displayed for the event (On or OFF) to happen.

- Push and release M+ until desired minutes are displayed.

- Push and release  ON/OFF/EVENT button to choose the next event to set or change.

- Push and release MODE button to choose AUTO, AUTO RANDom, or MANual operation.

- Replace clear cover.